Relation to Roadway

1998 and later

01 - On Roadway

The Roadway is that part of a trafficway designed, improved and ordinarily

used for motor vehicle travel or, where various classes of motor vehicles are

segregated, that part of a trafficway used by a particular class. Separate

roadways may be provided for traffic going in opposite directions, for example

northbound and southbound traffic or for trucks and automobiles. The

roadway and any shoulder alongside the roadway together make up the

roads.

02 - Shoulder

The Shoulder is that part of a trafficway contiguous with the roadway for

emergency use, for accommodation of stopped vehicles and for lateral

support of the roadway structure. A shoulder is a design feature of a road and

may or may not be paved.

03 - Median

A Median is defined as that area of a divided trafficway between parallel roads

separating the travelways for traffic in opposite directions. The principal

functions of a median are to provide the desired freedom from interference of

opposing traffic, to provide a recovery area for out-of-control vehicles, to

provide a stopping area in case of emergencies, to provide for speed change

and storage of left-turning and U-turning vehicles, and to minimize headlight

glare. Medians may be depressed, raised or flush. Flush medians can be as

little as 4-feet wide between roadway edgelines. Painted roadway edgelines

4 feet wide or wider denote medians. Medians of lesser width must have a

barrier to be considered a median. Medians do not include “shoulders” and

“separators.”

04 - Roadside

The Roadside refers to a location off the roadway, but inside the right-of

way. It is the outermost part of the trafficway, which lay between the outer

property line or other barrier and the edge of the first road encountered in the

trafficway.

05 - Outside Trafficway/Outside Right-of-way

06 - Off Roadway - Location Unknown

Off Roadway - Location Unknown refers to a location off the roadway, but its

relationship to the right-of-way is not known.

07 - In Parking Lane

In Parking Lane refers to a strip of road located on the roadway, or next to the

roadway, on which parking is permitted. This includes curb-side and edge-of

roadway parking (for example, legal residential parking, city street parking,

etc.).Sometimes a strip of roadway can be designated for parking at certain

hours of the day (parking lane) and for regular travel at other hours (travel

lane). This element should not be used during hours when parking is NOT

permitted.

08 - Gore (See figure on page 49)

A gore is an area of land where two roadways diverge or converge. The area

is bounded on two sides by the edges of the roadways, which join at the point

of divergence or convergence. The direction of traffic must be the same on

both of these roadways. The area includes shoulders or marked pavement if

any, between the roadways. The third side is 60 meters (approximately 200

feet) from the point of divergence of convergence or, if any other road is within

70 meters (230 feet) of that point, a line 10 meters (33 feet) from the nearest

edge of such road.

Gore Inclusions:

- Areas at rest area entry or exit ramps.

- Areas at truck weight station entry or exit ramps.

- Areas where two main roadways diverge or converge.

- Areas where a ramp and another roadway, or two ramps, diverge or

converge.

- Areas where a frontage road and another roadway or two frontage

roads, diverge or converge.

- And others.

Gore Exclusions:

- Islands for channelization of vehicle movements.

- Islands for pedestrian refuge.

10 - Separator

A Separator is the area of a trafficway between parallel roads separating

travel in the same direction or separating a frontage road from other roads

(see Trafficway Diagram in this section). A Separator may be a physical

barrier or a depressed, raised, flush, or vegetated area between roads.

11 - Two-way Continuous Left-turn Lane (since 2001)

A Two-Way Continuous Left-Turn Lane is for left-turning vehicles in both

directions of travel. Vehicles stopped or in motion within a continuous left-turn

lane are considered to be in transport, and should be included.

(See Trafficway Flow)

Relation to Roadway (Continued)

1975 to 1997

Variable = REL_ROAD

Element = 1 - On Roadway

2 - Shoulder

3 - Median

4 - Roadside

5 - Outside Right-of-way

6 - Off Roadway - Location Unknown

7 - In Parking Lane (since 1980)

8 - Gore (since 1982)

9 - Unknown